# APPENDIX 5

# INSTRUMENTS SPECIFICATIONS

## The single axis rate gyroscope

| Characteristic | Value |
| --- | --- |
| Angular velocity Range | 0.2 to 100deg/s or 0.1 to 50 deg/s |
| Angle range | +/- 45o max. |
| Roll, Pitch | +/- 45o max. |
| Yaw | +/- 180o max. |
| Accuracy | +/- 1 % FS |
| Bias drift | +/-2 deg/ Hr. |
| Resolution | 0.1 deg. |
| Operating Temperature | -20 to +70 OC |
| Bandwidth | 10 Hz |
| Dimansions | 75.6 x 42.5 x 65 mm |

## The three axis accelerometer

## The hand steering wheel potentiometer

RS Spectral 534 Series  200 ref 173-366
22 mm 3W 10 turn wirewound
 - resistance tolerance  5 %
- Linearity (independent)  0.25 %
- Mechanical rotation 3600 +10 -0
- Starting torque 36 gcm
- Operating temperature -55C to 125C
- 22.22  L 19.1
- Shaft 6.35  L 20.6
- Mounting bush 9.5  L 35

## Power supply converters:

-     RS 5 w Regulated Dual output NMX SO Series. Model: NMXD 1205. SO 12.   Voltage output is 5V and the maximum current output is 500 mA.

-    RS Vehicle power adaptor (12V DC to 230V AC). Model SM 1791.Provides 230V rms quasi-sine wave output suitable for use with equipment fitted with a mains transformer. It is a 120 w Battery to Mains Converters, with a input voltage between 11 and 32V DC and a 230V AC output voltage.